#### Allen-Bradley Beginning, Intermediate and Advance PLC Functions

##### Max Rabiee, Ph.D., P.E.

Associate Professor

Electrical and Computer Engineering Technology (ECET)

College of Applied Science (CAS)

##### University of Cincinnati

##### Table of Contents

Description of Workshop							2

Exercise #1	Programming PLC Station				3

Exercise #2	Programming PLC Station (Continued)		7

Exercise #3	Latch/Unlatch functions				13

Exercise #4 	Using the Force Function				16

Exercise #5	Timer Functions						18

Exercise #6	Counter Functions					24

Exercise #7	Math, Compare, and Jump Functions		30

Exercise#8	Sequencer Functions					33

Description of Workshop

During this workshop, we will discuss in detail PLC selection, port connections, relay logic, and ladder logic rules.  We will cover the method of programming Allen-Bradley Programmable Logic Controllers (PLCs).  Each participant will use individual PLC simulation software to write ladder logic programs for process and industrial control systems.

The workshop participants will learn the items in the following list.

1.	PLC ladder diagram rules.  Contacts, Relays, and Switches in a PLC system. Use of Normally Open (NO), and Normally Closed (NC) contacts, and switches in order to program a PLC for an industrial control system. Learn the use of Latch and Unlatch functions.

2.  Learn the use of timer relays, On-Delay timer contacts, and Off-Delay timer contacts.  Discuss the difference between Retentive Timer, and Non-retentive Timer functions.Use of Counter relays, and Counter Contacts. Discuss the difference between Counter Up, and Counter Down functions.  Learn the use of Move, and Jump functions in a PLC ladder diagram.

- Discuss the use of PLC Skip, Jump, Master Control Relay (MCR), Move, Math, and Sequencer functions.  Explain the use of subroutines in a PLC ladder diagram. Discuss and use Programmable Logic Controller COMPARE functions.  Discuss and use Programmable Logic Controller ADD, SUBTRACT, MULTIPLY, and DIVIDE functions. Discuss and use Programmable Logic Controller AND, OR, NOR, NAND, and XOR functions.

4.	Discuss and use the Sequencer Output (SQO), Sequencer Input (SQI), Sequencer Compare (SQC) functions. Learn how to connect PLC Sequencer functions in order to have more output (i.e., Parallel connection).  Learn how to cascade the PLC sequencer functions in order to have more steps.

Exercise NO. 1

(Programming PLC Station)

1.	In the following relay logic diagram the green light would turn on when switch number one (SW1) is pressed, while the red light would turn on when both switches number one and two are closed.

Figure 1-   Relay Logic Diagram

Step 1)	Assign the Input and Output terminals.

Input Device			PLC Input Port Address

Switch one (SW1)

Switch two (SW2)

Output Device		PLC Output Port Address

# Green Pilot Light (GPLT)

Red Pilot Light (RPLT)

Step 2)	Figure 2 displays a PLC Input/output port connection diagram example.  Draw the PLC I/O port connection diagram of the relay logic diagram in Figure 1.

Figure 2- PLC I/O Connection Diagram

Step 3)	Use the LogixPro simulation software to create the PLC ladder logic diagram of the relay logic diagram displayed in Figure 1.  Draw the PLC ladder logic diagram.

Step 4)	Download the PLC ladder logic diagram.  Place the PLC in Run/Monitor mode.

2.	Figure 3 displays a relay logic diagram.

Figure 3- Relay Logic Diagram

Step 1)	Describe the operation of the control system displayed in Figure 3.

Step 2)		Assign the Input and Output terminals as illustrated below.

Input Device			PLC Input Port Address

## Pushbutton one (PB1)

Pushbutton two (PB2)

Limit Switch one (LS1)

Output Device		PLC Output Port Address

Motor one (M1)

Red Pilot Light (RPLT)

Step 3)		In the blank space below, draw the PLC input/output port connection for the I/O address assignments in step 2.  Then, complete the PLC I/O connection on your PLC lab station.

Step 4)		Use the LogixPro software to create the PLC ladder logic diagram for the relay logic diagram displayed in Figure 3.  Draw the PLC ladder logic diagram below.

Step 5)		Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

Exercise NO. 2

(Programming PLC Station Continued)

1.	Figure 1 displays a relay logic diagram.

Figure 1- Relay Logic Diagram

Step 1)	Describe the operation of the control system displayed in Figure 1.

Step 2) 	Assign the Input and Output terminals as illustrated below.

Input Device			PLC Input Port Address

Switch One (SW1)

# Stop Pushbutton  (Stop)

Limit Switch One (LS1)

Output Device		PLC Output Port Address

Motor one (M1)

Red Pilot Light (RPLT)

Step 3)	In the blank space below, draw the PLC input/output port connection for the I/O address assignments in step 2.  Then, complete the PLC I/O connection on your PLC lab station.

Step 4)	Use the LogixPro software to create the PLC ladder logic diagram for the relay logic diagram displayed in Figure1.  Draw the PLC ladder logic diagram below.

Step 5)	Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

2.	Figure 2 displays a relay logic diagram.

Figure 2- Relay Logic Diagram

Step 1)		Describe the operation of the control system displayed in Figure 2.

Step 2)	Assign the Input and Output terminals.

Input Device			PLC Input Port Address

Pushbutton  (PB)

Thermostat  (TSW)

Output Device		PLC Output Port Address

Motor one (M1)

Green Pilot Light (GPLT)

Red Pilot Light (RPLT)

Step 3)	    In the blank space below, draw the PLC input/output port connection for the I/O address assignments in step 2.  Then, complete the PLC I/O connection on your PLC lab station.

Step 4)	     Use the LogixPro software to create the PLC ladder logic diagram for the relay logic diagram displayed in Figure 2.  Draw the PLC ladder logic diagram below.

Step 5)	    Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

- Use the LogixPro to draw the following relay logic diagram.

Describe the operation of the circuit:

4.	Use the LogixPro to draw the following relay logic diagram.

Describe the operation of the circuit:

Exercise NO. 3

(Latch/Unlatch functions)

- Use the LogixPro to draw the following relay logic diagram.

- Describe the operation of the circuit:

- Use the LogixPro to draw the following relay logic diagram.

Describe the operation of the circuit:

3.	Draw the PLC ladder diagram for a circuit that carries out the following control operations.

A)	When switch one (SW1) is closed, the green light turns on.

B)	When switch two (SW2) is closed, the yellow light turns on.

C)	When both SW1 and SW2 are closed, the green and yellow lights would be off, and only the red light would turn on.

4.	Draw the PLC ladder diagram for a circuit that operates as follow:

A)	The B-SW switch is the emergency stop switch.

B)	When PBR is pressed, red light and motor number one are energized.  They will stay on until the B-SW switch is opened.

C)	When PBG is closed, both white and green light come on, and both motors will run.  They will stay on until the B-SW switch is opened.

Exercise NO. 4

(Using the Force Function)

- In the control system displayed below, once the start push button is pressed, the motor will energize and the relay M-AUX will close.  The motor will run until the stop switch is opened.  Use the FORCE condition to carry out the operation.

Ladder Logic Diagram:

2.	In the following example motor number one (M1) will run when switch number one SW1) 	is closed, and motor number two (M2) will run when switch number two (SW2) is closed. Once either motor starts, it will run until the corresponding stop switch is opened.

Use the FORCE condition to carry out the operation.

Ladder Logic Diagram:

Exercise NO. 5

(Timer Functions)

Allen-Bradley SLC-500 PLCs have three different types of timers.

1.	Non-retentive, Timer On Delay (TON)

2.	Retentive, Timer On Delay (RTO)

3.	Non-retentive, Timer Off Delay (TOF)

### 1.	Non-retentive, Timer On Delay (TON)

The contact will close (for N.O. contact), or open (for N.C. contact) after a specific delay time if the timer coil is energized. The contact will immediately open or close if the coil is de-energized by opening the enable/run switch.

2.	Retentive, Timer On Delay (RTO)

The contact will close (for N.O. contact), or open (for N.C. contact) after a specific delay time if the timer coil is energized. The Reset function (RES) must be utilized to reset the timer.

3)	Non-retentive, Timer Off Delay (TOF)

The contact will close (for N.O. contact), or open (for N.C. contact) after a specific delay time if the timer coil is de-energized.  The contact will immediately open or close if the coil is energized by closing the enable/run switch.

- Program your PLC station to for a control circuit that carries out the following sequence of events.

(A)	When SW2 is closed, the red light turns on, and Motor number one (M1) would start running, and the bell would sound.

(B)	After ten (10) seconds Motor number one (M1) would stop running, and the White light would turn on.

(C)	Fifteen (15) seconds after the processes in step number two have been carried out, both the Red, and the White lights should turn off, and the Green light should turn on.

(D)	Finally, five (5) seconds after the processes in step number three have been carried out, both motors should start running.

(E)	If SW1 is opened, then the process must be halted.

Step 1)	Draw the relay logic diagram

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper                              connections,  and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.   Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

2.	Program your PLC station to for a control circuit that carries our the following operations.

When switch SW1 is closed the Red, White, and Green pilot lights turn on and off every two seconds.  That is, the Red light comes on for two seconds and then goes off.  Then the white light comes on for two seconds and goes off.  Finally, the green light should come on for two seconds.  This process should continue until the SW1 switch is opened.

Step 1)		Draw the relay logic diagram

Step 2)		Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

3.	Program your PLC station to for a control circuit that carries our the following operations

When the start push button is depressed, motor number one turns on.  After five (5) seconds, motor number two turns on.  Ten (10) seconds after motor number two turns on, the Red pilot light should light. The process should continue until the master stop switch is opened.

Step 1)		Draw the relay logic diagram

Step 2)		Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

4.	Program your PLC station to for a control circuit that carries out the following sequence of events to surface harden steel plats in a factory.

A)	A normally open switch is closed to start the process.  The Green pilot light indicates the start of the process.

B)	When the limit switch is closed, Motor number one will run for five seconds, to lift the part.  Use the thermostats on the PLC stations in the Lab for limit switches.

C)	Next, the Red light lights for ten seconds to indicate the induction heating process.

D)	After the red light has been on for ten seconds, turn it off and turn  Motor number two on for eight seconds to spray cold water on the part.

E)	Two second after motor number two shuts off, the white light should turn on.

Step 1)		Draw the relay logic diagram

Step 2)		Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

###### Exercise NO. 6

(Counter Functions)

Allen-Bradley SLC-500 PLCs have two different types of timers.

1.	Counter-Up (CTU)

2.	Counter-Down (CTD)

Notice that we must use the reset function (RES) to reset the counters.

1.	Program your PLC station to for a control circuit that carries our the following operations

A fan, is to be turned on when counter A goes from 7 down to 0 and when either counter B goes up to 14 or counter C has gone all the way from 14 down to 0.  One switch resets the entire process.

Step 1)		Draw the relay logic diagram

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

2.	Using the PLC Counter  and Retentive Timer function, program your PLC station using timer off-delay functions for a control circuit that carries out the following sequence of events.

A)	If the Red push button is pressed six (6) times, turn the red light on.

B)	If the Green push button is pressed three (3) times, turn the Green light on

C)	If the Red push button and the Green push button are pressed twelve (12) times, turn only the white light on.

Note that, as long as the total is twelve, the white light should turn on.  Which means the pressing buttons combination does not matter.

Three Timers should show how long the each light has been on.

Step 1)	Draw the relay logic diagram

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

3.	Program your PLC station to for a control circuit that carries our the following operations.

A)	Use the SW switch for reset.  If we press the Green push-button once, Red				and Green lights will turn on.

B)	If we press the Green push-button twice, only the white light will turn on.

Step 1)		Draw the relay logic diagram

Step 2)		Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

4.	Program your PLC station to for a control circuit that carries our the following operations.

A)	Use the SW switch to reset the system.

B)	If we press push-button green, and push-button red three times each, the Red and Green lights will turn on.

C)	The lights will stay on for only three (3) seconds.

Step 1)		Draw the relay logic diagram

Step 2)		Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

Exercise NO. 7

(Math, Compare, and Jump Functions)

1.	Program your PLC lab station to operate as follow:

A)	When the total count from pushbuttons red and green is equal to 10, only the red light will turn on.

B)	When the total count is between 10 and 15, only the green light will turn on.

C)	When the total count is greater than 15, only the white light will turn on.

D)	If the difference between the red and green pushbutton counts is greater than 6, 			the bell will sound continuously.

E)	Use the black switch as master reset switch.

Step 1)	Draw the relay logic diagram

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

- Program your PLC lab station to operate as follow:

A)	If the result of multiplying red, green, and  pushbuttons is greater than or equal 60, 				then all three lights (Red, Green, and White) will turn on.

B)	If the result of dividing red pushbutton count by three is greater than five, then the bell will sound five times.

Step 1)	Draw the relay logic diagram

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections, and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

3.	Devise an MCR system to control the assembly line shown below.  All six (6) stations are to function as set up by one of the two registers.  A switch detects the presence of a part on the assembly line.  Short and tall parts are sent down the line.  Short  parts get all six operations.  The tall parts will only get the first operation,   the forth operation,  the fifth operation, and the sixth operation.  Tall parts are detected by a limit switch (e.g., use the Thermostat switch on our PLC stations).  This limit switch should activate the MCR function and seal it.  After the sixth operation has been carried out, the MCR function should be unsealed, and all the operations must turn off until another part is detected again.

Operations:	one	two	three	four	five	six

Pattern A:	1	1	1	1	1	1

Pattern B:	1	0	0	1	1	1

Note: There should be two second delays between starting of  each operation.  Each operation  is described below.

One		Turns  on Motor number one.

Two		Turns on Motor number two.

Three		Turns on the Green light.

Four		Turns on the White light.

Five		Turns on the red light.

Six		Sounds the bell.

Step 1)	Draw the relay logic diagram

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

Exercise NO. 8

(Sequencer Functions)

Allen-Bradley SLC-500 PLCs have three different types of Sequencer functions.

1.	Sequencer Output (SQO)

2.	Sequencer Compare (SQC)

3.	Sequencer Load (SQL)

##### Example 1- Sequencer Output (SQO) with Manual Enable/Step Input

Example 2- Sequencer Output (SQO) with Timer-control Enable/Step Input

Example 3- Sequencer Compare (SQC)

Example 4- Sequencer Load (SQL)

1.	Use the SEQUENCER function to place the following machine matrix in the PLC.  Each step should last two seconds. The process should continue until a Master Stop switch is opened.

Step Number		M1	M2	G-PLT	R-PLT	W-PLT	BELL

1	off	on	off		off		off		off

2	off	off	on		on		off		off

3	off	off	off		on		on		off

4	on	on	on		on		on		on

5	on	on	off		off		off		off

6	off	off	off		off		off		off

Step 1)	Draw the relay logic diagrams.

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

2.	Use two sequencer and one timer function to run your PLC lab station to do the 	following sequence of events.

Step 1			Step 2			Step 3			Step 4

Event:	Run Motor One	Turn on Red and	Run Both Motors	Turn on the

Green Lights						 White Light

Duration:	2 Seconds		2.5 Seconds		3.25 Seconds		1.5 Seconds

Step 1)	Draw the relay logic diagrams

Step 2)		Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)		Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.

3.	Use the sequencer functions to create an operation scheme for a dishwasher.  The six operating steps and the delay time between the steps are listed below.

Energize the soap release solenoid (e.g., turn green light on), for four seconds.

Open the input valve for hot water (e.g., turn white light on ), for five seconds.

Operate the washer impeller  (e.g., turn motor number one on), for  twelve seconds.

Open the rain water valve (e.g., ring the bell).

Turn on the drain pump (e.g., turn on motor number two), for three seconds.

Turn on the heat element during the drying cycle (turn on the red light), for six seconds.

Step 1)	Draw the relay logic diagrams

Step 2)	Assign PLC  Input/Output ports to the devices. Make the proper connections,  and program your PLC to carry out the above logic.

Step 3)	Use the LogixPro software to create the PLC ladder logic diagrams.  Download the PLC ladder logic diagram into the PLC station.  Place the PLC in Run/Monitor mode.